# How to make an update from V5 to V6

- Install  Checkbox-Configurator Version 2.7 Release 0: 
Execute "setup.exe" and follow the instructions.
Note: If you use WindowsNT as your operating system installation permissions are required.
If you don’t have additional permissions to change the appearance of your desktop and to customize the Start menu, error messages will occur.
Go on with the installation routine. Only the Start menu and your desktop wont be updated.

- Switch on Checkbox.
Start new Configurator Software with level 2 ( password: engage )

- Click the Download Box Operating System to Checkbox ... command in the File menu. Clicking this command displays the Open dialog box. Choose update.bin and click OK.

- Switch off Checkbox when the following message appears.



Press STATUS button while switching on Checkbox.
After a few seconds the LCD TOLERANCE display shows the word ‘LOAd’.

- Click the Download Box Operating System to Checkbox ... command and choose one of the following files: i_conf10.bin, c_conf10.bin, ib_601.bin and cb_601.bin
Attention: You must not use any *.bin files for download which are older than files on this disk.

- As soon as the following message appears don’t click OK, but switch off / on Checkbox  first, then quit the message with OK.



Download of data to Checkbox will continue.

- Press OK to quit the message. After a few seconds the following message appears:

- Make sure, that Configure Checkbox with default values is active. Press OK.

- Successful update will be indicated by the following message. Click OK.

- If LCD display shows Error-message "E 42" after pressing START button then press STOP button and click Download Teach Data to Checkbox command in the File menu.
Download the File zero.typ to all types. You can find this file on the installation disk.

- After pushing the Start button of the Checkbox Error-message "E 40" appears, which means that the teach data is not correct.

- Select the "Teach-Mode" with the key switch and teach dummy parts to all types.

- The Checkbox should now work correctly.